|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 1 // BEGINNER |
| --- | --- | --- |
| CARD_01 Traffic Light Build a traffic signal for a tiny road junction. // parts Arduino Uno · Breadboard · 3 LEDs (red, yellow, green) · 3 × 220 Ω resistors // pins Red LED → pin 8 ¦ Yellow LED → pin 9 ¦ Green LED → pin 10 // your_code_must [ ] Only ONE LED is on at a time [ ] Order is Red → Yellow → Green, then repeats forever [ ] Each LED stays on for about 1 second time_limit = 5 min name: __________________ time: _____ pass [ ] |  |  |

✂  cut here

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 1 // BEGINNER |
| --- | --- | --- |
| CARD_02 Button Switch Make a push-button switch that controls a light. // parts Arduino Uno · Breadboard · 1 LED · 1 × 220 Ω resistor · 1 pushbutton // pins LED → pin 13 ¦ Button → pin 2 // your_code_must [ ] LED is ON only while the button is held down [ ] LED turns OFF as soon as the button is released [ ] Works again and again (not just once) time_limit = 5 min name: __________________ time: _____ pass [ ] |  |  |

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 1 // BEGINNER |
| --- | --- | --- |
| CARD_03 SOS Signal A stranded robot needs help! Make it send the SOS signal with light. // parts Arduino Uno · 1 LED · 1 × 220 Ω resistor // pins LED → pin 13 // your_code_must [ ] 3 short blinks, then 3 long blinks, then 3 short blinks [ ] A long blink is about 3 times as long as a short one [ ] Short pause, then the whole pattern repeats time_limit = 5 min name: __________________ time: _____ pass [ ] |  |  |

✂  cut here

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 2 // INTERMEDIATE |
| --- | --- | --- |
| CARD_04 Parking Sensor Build a car parking sensor that beeps faster as the wall gets closer. // parts Arduino Uno · Breadboard · Ultrasonic distance sensor (4-pin) · Piezo buzzer // pins Trig → pin 7 ¦ Echo → pin 6 ¦ Buzzer → pin 8 // your_code_must [ ] Silent when the object is farther than 30 cm [ ] Beeps when the object is 30 cm or closer [ ] The closer the object, the faster the beeping [ ] Test by dragging the object in the simulator time_limit = 10 min name: __________________ time: _____ pass [ ] |  |  |

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 2 // INTERMEDIATE |
| --- | --- | --- |
| CARD_05 Night Light Make a street light that switches on by itself when it gets dark. // parts Arduino Uno · Breadboard · Photoresistor (LDR) · 10 kΩ resistor · 1 LED · 1 × 220 Ω resistor // pins LDR sensor → pin A0 ¦ LED → pin 13 // your_code_must [ ] LED is ON when it is dark [ ] LED is OFF when it is bright [ ] Reacts live when the light level is changed in the simulator time_limit = 8 min name: __________________ time: _____ pass [ ] |  |  |

✂  cut here

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 2 // INTERMEDIATE |
| --- | --- | --- |
| CARD_06 Smart Door Build an automatic door that opens for a visitor and closes by itself. // parts Arduino Uno · Breadboard · Micro servo · 1 pushbutton // pins Servo → pin 9 ¦ Button → pin 2 // your_code_must [ ] Door (servo) rests closed at 0° [ ] Pressing the button opens it to 90° [ ] It stays open for about 3 seconds, then closes back to 0° time_limit = 10 min name: __________________ time: _____ pass [ ] |  |  |

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 3 // CHALLENGE |
| --- | --- | --- |
| CARD_07 Intruder Alarm Protect the lab! Build an alarm that catches anyone who gets too close. // parts Arduino Uno · Breadboard · Ultrasonic sensor · LED + 220 Ω resistor · Piezo buzzer · Pushbutton // pins Trig → 7 ¦ Echo → 6 ¦ LED → 13 ¦ Buzzer → 8 ¦ Reset → 2 // your_code_must [ ] Normal state: LED and buzzer are off [ ] Object closer than 15 cm → LED blinks AND buzzer sounds [ ] Alarm KEEPS going even if the object moves away [ ] Alarm stops only when the reset button is pressed time_limit = 12 min name: __________________ time: _____ pass [ ] |  |  |

✂  cut here

|  | IUBPC ROBOTICS · PROBLEM SOLVING CHALLENGE | LEVEL 3 // CHALLENGE |
| --- | --- | --- |
| CARD_08 Reaction Game How fast are your reflexes? Build a game that measures it. // parts Arduino Uno · 1 LED + 220 Ω resistor · 1 pushbutton // pins LED → pin 13 ¦ Button → pin 2 // your_code_must [ ] LED starts OFF, then turns ON after a random 2–5 second wait [ ] Player presses the button as fast as possible [ ] Serial Monitor prints the reaction time in milliseconds [ ] The game repeats automatically time_limit = 12 min name: __________________ time: _____ pass [ ] |  |  |

|  | JUDGE_SHEET IUBPC Robotics Challenge · do not show to participants |
| --- | --- |

// quick_rules

>  Hand out ONE challenge card at a time; start the timer when they begin.

>  Maximum 2 hints per attempt: Hint 1 = wiring/pin tip, Hint 2 = code skeleton.

>  A card is PASSED only if every [ ] on the card is satisfied when you press Start Simulation.

>  Prize for any solve; bigger prize for the fastest solver or any Level 3 solve.

>  Always write the participant name and email on the sign-up sheet, pass or fail.

CARD_01  Traffic Light   LEVEL 1 // BEGINNER

hint_1: Each LED needs its own pin set as OUTPUT, and a resistor in series.

hint_2: Use digitalWrite(pin, HIGH), delay(1000), digitalWrite(pin, LOW) for each LED in order.

void setup() {

pinMode(8, OUTPUT);

pinMode(9, OUTPUT);

pinMode(10, OUTPUT);

}

void loop() {

digitalWrite(8, HIGH);  delay(1000);  digitalWrite(8, LOW);

digitalWrite(9, HIGH);  delay(1000);  digitalWrite(9, LOW);

digitalWrite(10, HIGH); delay(1000);  digitalWrite(10, LOW);

}

CARD_02  Button Switch   LEVEL 1 // BEGINNER

hint_1: Wire the button between pin 2 and GND, and set the pin mode to INPUT_PULLUP.

hint_2: With INPUT_PULLUP a pressed button reads LOW. Check digitalRead(2) inside loop().

void setup() {

pinMode(2, INPUT_PULLUP);

pinMode(13, OUTPUT);

}

void loop() {

if (digitalRead(2) == LOW) digitalWrite(13, HIGH);

else digitalWrite(13, LOW);

}

CARD_03  SOS Signal   LEVEL 1 // BEGINNER

hint_1: Write one helper function that blinks once for a given duration.

hint_2: Short ≈ 200 ms, long ≈ 600 ms. Call the helper 3 times per group with a for loop.

void blinkMs(int on) {

digitalWrite(13, HIGH); delay(on);

digitalWrite(13, LOW);  delay(200);

}

void setup() { pinMode(13, OUTPUT); }

void loop() {

for (int i = 0; i < 3; i++) blinkMs(200);

for (int i = 0; i < 3; i++) blinkMs(600);

for (int i = 0; i < 3; i++) blinkMs(200);

delay(2000);

}

CARD_04  Parking Sensor   LEVEL 2 // INTERMEDIATE

hint_1: Send a 10 µs HIGH pulse on Trig, then use pulseIn() on Echo. Distance (cm) = time × 0.034 / 2.

hint_2: Use tone(8, 1000, 50) then delay(). Make the delay shorter when the distance is smaller (try map()).

long readCm() {

digitalWrite(7, LOW);  delayMicroseconds(2);

digitalWrite(7, HIGH); delayMicroseconds(10);

digitalWrite(7, LOW);

long t = pulseIn(6, HIGH, 30000);

if (t == 0) return 999;

return t * 0.034 / 2;

}

void setup() {

pinMode(7, OUTPUT); pinMode(6, INPUT); pinMode(8, OUTPUT);

}

void loop() {

long d = readCm();

if (d > 30) { noTone(8); delay(100); }

else {

tone(8, 1000, 50);

delay(map(constrain(d, 2, 30), 2, 30, 50, 500));

}

}

CARD_05  Night Light   LEVEL 2 // INTERMEDIATE

hint_1: Build a voltage divider: LDR from 5V to A0, and 10 kΩ from A0 to GND. Read it with analogRead(A0).

hint_2: Print analogRead values to the Serial Monitor in light and in dark, then pick your threshold between them.

// Threshold 500 may need adjusting in the simulator

void setup() { pinMode(13, OUTPUT); }

void loop() {

int v = analogRead(A0);

if (v < 500) digitalWrite(13, HIGH);   // dark

else digitalWrite(13, LOW);            // bright

delay(50);

}

CARD_06  Smart Door   LEVEL 2 // INTERMEDIATE

hint_1: Add #include <Servo.h>, create a Servo object, and call attach(9) in setup().

hint_2: If the button is pressed: servo.write(90); delay(3000); servo.write(0);

#include <Servo.h>

Servo door;

void setup() {

pinMode(2, INPUT_PULLUP);

door.attach(9);

door.write(0);

}

void loop() {

if (digitalRead(2) == LOW) {

door.write(90);

delay(3000);

door.write(0);

}

}

CARD_07  Intruder Alarm   LEVEL 3 // CHALLENGE

hint_1: Reuse the distance function from the Parking Sensor, and keep the alarm state in a boolean variable.

hint_2: Set alarmOn = true when distance < 15. Set it to false only when the button is pressed.

bool alarmOn = false;

long readCm() {

digitalWrite(7, LOW);  delayMicroseconds(2);

digitalWrite(7, HIGH); delayMicroseconds(10);

digitalWrite(7, LOW);

long t = pulseIn(6, HIGH, 30000);

if (t == 0) return 999;

return t * 0.034 / 2;

}

void setup() {

pinMode(7, OUTPUT); pinMode(6, INPUT);

pinMode(13, OUTPUT); pinMode(8, OUTPUT);

pinMode(2, INPUT_PULLUP);

}

void loop() {

if (readCm() < 15) alarmOn = true;

if (digitalRead(2) == LOW) alarmOn = false;

if (alarmOn) {

digitalWrite(13, HIGH); tone(8, 1000); delay(150);

digitalWrite(13, LOW);  noTone(8);     delay(150);

} else {

digitalWrite(13, LOW); noTone(8);

}

}

CARD_08  Reaction Game   LEVEL 3 // CHALLENGE

hint_1: Use random(2000, 5001) for the wait, and millis() to measure time.

hint_2: Save millis() when the LED turns on. After the button press, print millis() minus the saved value.

void setup() {

Serial.begin(9600);

pinMode(13, OUTPUT);

pinMode(2, INPUT_PULLUP);

randomSeed(analogRead(A0));

}

void loop() {

digitalWrite(13, LOW);

delay(random(2000, 5001));

digitalWrite(13, HIGH);

unsigned long start = millis();

while (digitalRead(2) == HIGH) { }

Serial.print("Reaction time: ");

Serial.print(millis() - start);

Serial.println(" ms");

digitalWrite(13, LOW);

delay(2000);

}