|  | IUBPC ROBOTICS · FINAL BOSS CHALLENGE | BOSS // EXTREME |
| --- | --- | --- |
| SOLVE IT → JOIN IUBPC FOR FREE Free club entry pass for the one who cracks it. One attempt per person. |  |  |
| CARD_BOSS THE VAULT Build a smart vault door that only opens for the right secret knock. Most people will not crack it. // parts Arduino Uno · Breadboard · Micro servo · Pushbutton · Ultrasonic sensor (4-pin) · Red LED · Green LED · 2 × 220 Ω resistors · Piezo buzzer // pins Btn→2 ¦ Servo→9 ¦ Red→13 ¦ Green→12 ¦ Buzz→8 ¦ Trig→7 ¦ Echo→6 // secret_knock S – S – L – S S = press ≤ 0.3 s · L = press ≥ 0.6 s · in-between = WRONG // your_code_must [ ] 1. Starts LOCKED: servo at 0°, red LED ON, green LED OFF. [ ] 2. The button counts ONLY while a hand is within 20 cm of the ultrasonic sensor. Otherwise it is ignored. [ ] 3. The code is checked after the 4th press. A gap of more than 2 s between presses cancels the entry (no penalty). [ ] 4. Right knock → green ON, red OFF, servo 90°, two rising beeps. After 5 s it re-locks by itself. [ ] 5. Wrong knock → one long, low buzzer tone. [ ] 6. 3 wrong knocks in a row → 10 s LOCKOUT: red LED blinks fast, ALL input ignored, Serial Monitor counts down. [ ] 7. NO delay() in your main logic. Use millis(), so LED blinking and timing never freeze the button. [ ] 8. Debounce the button, and log every event (each press, unlock, wrong knock, lockout, relock) to the Serial Monitor. // rules time_limit = 45 min · hints = NONE · AI tools = NOT allowed The judge runs live tests on your simulation. One failed test = not passed. name: ____________________ time: _______ pass [ ] |  |  |

|  | JUDGE_SHEET · THE VAULT Boss card solution + live test script · do not show to participants |
| --- | --- |

// wiring

| Component | Connection |
| --- | --- |
| Pushbutton | One leg → pin 2; diagonally opposite leg → GND |
| Micro servo | Red → 5V; brown → GND; orange (signal) → pin 9 |
| Ultrasonic sensor (4-pin) | VCC → 5V; GND → GND; TRIG → pin 7; ECHO → pin 6 |
| Piezo buzzer | + → pin 8; − → GND |
| Red LED | Anode (+) → 220 Ω → pin 13; cathode (−) → GND |
| Green LED | Anode (+) → 220 Ω → pin 12; cathode (−) → GND |

// live_test_script (all must pass, first run)

1. Wake test: Drag the object FAR (over 20 cm), do the correct knock S-S-L-S. Nothing may happen.

2. Unlock test: Object NEAR (under 20 cm), knock S-S-L-S. Green on, red off, servo 90°, two rising beeps, Serial shows UNLOCKED. It re-locks by itself after about 5 s. Button is ignored while open.

3. Timing test: Knock with one press of about 0.5 s (in-between length): must be a WRONG KNOCK. Pause over 2 s in the middle of a knock: the entry must cancel, with no wrong-attempt penalty.

4. Lockout test: 3 wrong knocks in a row: red blinks fast for 10 s, Serial counts down, a correct knock during lockout is ignored, and normal operation returns after 10 s.

5. Code check: Search the code for "delay(": it must not appear (delayMicroseconds for the sensor is fine). Check that button changes are debounced with a millis() time check.

Passed = every test above succeeds in one live run. Winner gets the free club entry pass.

// solution_code (Arduino Uno)

// IUBPC Robotics Challenge - BOSS: The Vault

// Board: Arduino Uno (Tinkercad)

#include <Servo.h>

const int PIN_BTN = 2, PIN_TRIG = 7, PIN_ECHO = 6, PIN_BUZ = 8;

const int PIN_SERVO = 9, PIN_GREEN = 12, PIN_RED = 13;

const unsigned long DEBOUNCE = 30, SHORT_MAX = 300, LONG_MIN = 600;

const unsigned long GAP_MAX = 2000, OPEN_MS = 5000, LOCKOUT_MS = 10000;

const int WAKE_CM = 20, MAX_FAILS = 3;

enum State { LOCKED, OPEN, LOCKOUT };

State state = LOCKED;

Servo lockServo;

const char secret[4] = {'S', 'S', 'L', 'S'};

char entry[4];

int count = 0, fails = 0, beepStage = 0;

bool handNear = false, pressing = false;

bool rawLast = false, stable = false;

unsigned long lastChange = 0, pressStart = 0, lastRelease = 0;

unsigned long stateStart = 0, lastProx = 0, lastBlink = 0, lastSecPrint = 0;

long readCm() {

digitalWrite(PIN_TRIG, LOW);  delayMicroseconds(2);

digitalWrite(PIN_TRIG, HIGH); delayMicroseconds(10);

digitalWrite(PIN_TRIG, LOW);

long t = pulseIn(PIN_ECHO, HIGH, 30000);

if (t == 0) return 999;

return t * 0.034 / 2;

}

void logEvent(const char *msg) {

Serial.print("[");

Serial.print(millis());

Serial.print(" ms] ");

Serial.println(msg);

}

void setLocked() {

state = LOCKED;

lockServo.write(0);

digitalWrite(PIN_RED, HIGH);

digitalWrite(PIN_GREEN, LOW);

count = 0;

}

void checkEntry(unsigned long now) {

bool ok = true;

for (int i = 0; i < 4; i++) if (entry[i] != secret[i]) ok = false;

count = 0;

if (ok) {

state = OPEN;

stateStart = now;

beepStage = 0;

fails = 0;

lockServo.write(90);

digitalWrite(PIN_GREEN, HIGH);

digitalWrite(PIN_RED, LOW);

logEvent("UNLOCKED");

} else {

fails++;

tone(PIN_BUZ, 200, 800);

logEvent("WRONG KNOCK");

if (fails >= MAX_FAILS) {

state = LOCKOUT;

stateStart = now;

lastBlink = now;

lastSecPrint = now - 1000;   // print countdown immediately

fails = 0;

digitalWrite(PIN_RED, HIGH);

digitalWrite(PIN_GREEN, LOW);

logEvent("LOCKOUT START");

}

}

}

void onPress(unsigned long now) {

if (state != LOCKED || !handNear) return;   // ignored

pressing = true;

pressStart = now;

}

void onRelease(unsigned long now) {

if (!pressing) return;

pressing = false;

unsigned long d = now - pressStart;

char sym = (d <= SHORT_MAX) ? 'S' : (d >= LONG_MIN ? 'L' : 'X');

entry[count++] = sym;

lastRelease = now;

if (sym == 'S') logEvent("press: SHORT");

else if (sym == 'L') logEvent("press: LONG");

else logEvent("press: INVALID");

if (count == 4) checkEntry(now);

}

void setup() {

Serial.begin(9600);

pinMode(PIN_BTN, INPUT_PULLUP);

pinMode(PIN_TRIG, OUTPUT);

pinMode(PIN_ECHO, INPUT);

pinMode(PIN_BUZ, OUTPUT);

pinMode(PIN_RED, OUTPUT);

pinMode(PIN_GREEN, OUTPUT);

lockServo.attach(PIN_SERVO);

setLocked();

logEvent("VAULT READY");

}

void loop() {

unsigned long now = millis();

// proximity check every 100 ms

if (now - lastProx >= 100) {

lastProx = now;

handNear = (readCm() <= WAKE_CM);

}

// debounced button

bool raw = (digitalRead(PIN_BTN) == LOW);

if (raw != rawLast) { lastChange = now; rawLast = raw; }

if (now - lastChange >= DEBOUNCE && raw != stable) {

stable = raw;

if (stable) onPress(now); else onRelease(now);

}

// entry timeout: more than 2 s between presses cancels the attempt

if (state == LOCKED && count > 0 && !pressing && now - lastRelease > GAP_MAX) {

count = 0;

logEvent("entry cleared (timeout)");

}

if (state == OPEN) {

unsigned long e = now - stateStart;

if (beepStage == 0) { tone(PIN_BUZ, 880, 150); beepStage = 1; }

else if (beepStage == 1 && e >= 250) { tone(PIN_BUZ, 1320, 150); beepStage = 2; }

if (e >= OPEN_MS) { setLocked(); logEvent("RELOCKED"); }

}

if (state == LOCKOUT) {

unsigned long e = now - stateStart;

if (e >= LOCKOUT_MS) {

setLocked();

logEvent("LOCKOUT END");

} else {

if (now - lastBlink >= 100) {

lastBlink = now;

digitalWrite(PIN_RED, !digitalRead(PIN_RED));

}

if (now - lastSecPrint >= 1000) {

lastSecPrint = now;

Serial.print("Lockout: ");

Serial.print((LOCKOUT_MS - e + 999) / 1000);

Serial.println(" s left");

}

}

}

}